Tony Hong

1005109482

Oct. 28, 2020

# CSC258 Prelab (Lab 6)

Part 1:  Sequence Recognizer

- Given the starter circuit, the reset signal is an asynchronous reset that is active high. During tests, the reset should be used when we want to test a new sequence of 4 numbers without influence from the previous inputs.

- Below is the state table and assigned flip flop values for each state

| Input State | F2 | F1 | F0 | W (Input) | F2 | F1 | F0 | Output State |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 0 | 0 | 0 | 0 | 0 | 0 | A |
| A | 0 | 0 | 0 | 1 | 0 | 1 | 0 | B |
| B | 0 | 1 | 0 | 0 | 0 | 0 | 0 | A |
| B | 0 | 1 | 0 | 1 | 1 | 1 | 0 | C |
| C | 1 | 1 | 0 | 0 | 1 | 0 | 1 | E |
| C | 1 | 1 | 0 | 1 | 1 | 1 | 1 | D |
| D | 1 | 1 | 1 | 0 | 1 | 0 | 1 | E |
| D | 1 | 1 | 1 | 1 | 1 | 0 | 0 | F |
| E | 1 | 0 | 1 | 0 | 0 | 0 | 0 | A |
| E | 1 | 0 | 1 | 1 | 0 | 0 | 1 | G |
| F | 1 | 0 | 0 | 0 | 1 | 0 | 1 | E |
| F | 1 | 0 | 0 | 1 | 1 | 0 | 0 | F |
| G | 0 | 0 | 1 | 0 | 0 | 0 | 0 | A |
| G | 0 | 0 | 1 | 1 | 1 | 1 | 0 | C |

Note, that some of the transitions change more than 1 bit. It is impossible to design a state table for this FSM where each transition changes a maximum of 1 bit because there are 5 different states that either transition to or transition from E.

Below are the K-maps to determine the circuits that will implement the logic from the state table.

5. Below outlines a test plan for this FSM.

Prep: set clock tick to 1Hz, Async Reset the state and finally, turn on the Enable for the state.

Test 1: Turn W to 1 and watch 4 clock cycles pass and check if Z becomes 1. Let some more clock cycles pass and check if Z maintains its value of 1.

This test shows that the FSM detects the pattern 1111

Test2: Reset the state and turn W to 1 for 3 clock cycles. Turn W to 0 for the next clock cycle and turn W to 1 for the clock cycle after. Check to see that Z is 1 for that clock cycle

This test shows that the FSM detects the pattern 1101

Below are screen shots showing parts of Test 1.

Cycle 1:

Cycle 2:

Cycle 3:

Cycle 4:

Part 2:  FSM For CX2 + Bx + A

Below is a plan to complete the required computation along with 4-bit state representations for each step.

Below is a state table for all states of my controller and the signals for each ALU component

| State | ld_a | ld_b | ld_c | ld_x | alu_select_a | alu_select_b | ld_alu_out | alu_op | ld_r | next_state |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 0 | 0 | 1 | 0 | 00 | 00 | 0 | 0 | 0 | 0001 |
| 0001 | 0 | 0 | 0 | 1 | 00 | 00 | 0 | 0 | 0 | 0011 |
| 0011 | 1 | 0 | 0 | 0 | 00 | 00 | 0 | 0 | 0 | 0010 |
| 0010 | 0 | 1 | 0 | 0 | 00 | 00 | 0 | 0 | 0 | 0110 |
| 0110 | 0 | 1 | 0 | 0 | 01 | 11 | 1 | 1 | 0 | 0111 |
| 0111 | 0 | 1 | 0 | 0 | 01 | 00 | 1 | 0 | 0 | 0101 |
| 0101 | 1 | 0 | 0 | 0 | 11 | 11 | 1 | 1 | 0 | 0100 |
| 0100 | 1 | 0 | 0 | 0 | 00 | 10 | 1 | 1 | 0 | 1100 |
| 1100 | 0 | 0 | 0 | 0 | 00 | 01 | 0 | 0 | 1 | 1100 |

Test and Screenshots: C=4, X=3, B=6, A=2 Expected Output: 56

As seen in the final screenshot, the Output is indeed 56. (3*16 + 8)